Making a pen

1. Cut the blank to dimensions required.

2. Drill out the centre.

a. Rough surface of tube with sandpaper

b. Apply adhesive to outside of tube (could be superglue or epoxy)

3. insert the tube into the blank using a twisting motion. Ensure tube is inserted fully with edge just below the surface of the wood blank.

4. Square off the ends to make the blank the same length as the tube insert using a barrel trimmer -  if no barrel trimmer is available then (carefully) square off the ends on the disc sander. Only reduce to where the end of the brass tube is level with the end of the blank.

5. Mount the mandrel and mandrel saver on the lathe.

6. Fit blank to mandrill using the correct bushings for the pen kit. If making a kit that requires two wood blanks be sure you have marked the matching ends and mounted them with the alignment marks matching.

7. Set the lathe tool rest to the height and distance from the blank.

8. Set the lathe to the speed required for turning process.

9. Carefully slide the chisel at an angle down on the blank until it begins to cut.

10. Move the chisel evenly along the blank carefully removing the excess wood from the blank until the ends are the same dimensions as the bushings.

11. When almost down to the correct dimensions work on the shape that you intend the pens to finish up with... evenly tapered end to end, or sausage shaped, or torpedo shaped, and carefully turn to that shape.

12. Sand with 120 grit sandpaper to smooth the surface of the blank then work up through the grits until you reach 240 grit.

13. Apply sanding sealer to seal the blank, allow to dry.

14. Sand again beginning with 240 grit sandpaper to remove any visible scratches. (Perform sanding with lathe turned off and sanding along the longitudinal axis of the blank.)

15. Work through the grits to the desired level (400-500 grit) to further refine the finish.

16. Using thin CA glue (superglue) apply to the blank, spreading it with kitchen roll, before it has time to dry. (Apply at least 2 coats)

17. Sand again beginning with 240 grit and work up through the grits.

18. Apply medium viscosity super glue using kitchen roll.

19. Allow to dry then sand the surface as before to remove any streaks. (Apply at least two coats – or more if needed.)

20. Once all streaks have been removed use fine polishing paste, (Yorkshire Grit) to get smooth finish.

21. Polish off the wooden blank to a glaze finish.

22. Fit pen kit accessories using a vice to compress any necessary elements into position, making sure that wherever necessary you use the pen bushings to prevent damage to the finished pen body. Remember that the ends of the turned blank may not be the same diameter after turning. Be sure to orient the blank so the appropriate kit part is installed on the correct end.